HEPA VACUUM WASTE BAG REMOVAL-GLOVE BAG METHOD

- Place glove-bag on floor.

- Place new exhaust filter, micro filter, and main filter in upper area of glove-gab.

- Place small spray bottle containing water & cloth or paper wipes in upper area of bag.

- Disconnect hose from vacuum and seal ends with duct tape.

- Roll dirty HEPA vacuum into upper area of bag with uncoiled power cord extending onto floor outside of glove-bag.

- Seal top of glove-bag around power cord using duct tape.

- Open HEPA vacuum clamps and turn power on using built-in gloves within bag.

- Separate top and bottom of vacuum and empty heavier dust into bottom of glove-bag.

- Remove and replace exhaust filter, micro filter, and main filter.

- Place dirty filters in bottom area of glove-bag.

- Re-clamp top and bottom of vacuum together.

- Re-wipe outside of vacuum and upper are of glove-bag using spray bottle and cloth or paper wipes.

- Place dirty wipes in bottom area of glove-bag.

- Separate upper clean area from bottom dirty area within glove-bag by twisting the poly bag and wrapping twist with duct tape.

- Open upper area of glove-bag and remove vacuum.

- Turn power off and reconnect hose to vacuum.

- Dispose glove-bag as hazardous waste.